Building Vulnerability Assessment Report

Official Technical Record — Bhuj Earthquake Event (2001)

| Earthquake Event: | Bhuj, Gujarat Earthquake (26 January 2001 ¦ Mw 7.7 ¦ Depth ~17 km) |
| --- | --- |
| Assessment Framework: | NDMA Rapid Visual Screening (RVS) & Earthquake Field Reconnaissance |
| Identification Protocol: | Deterministic SHA-256 Hashing & Deduplication over Physical Structures |
| Report Date / Status: | October 17, 2026 ¦ Official Engineering Assessment |

# 1. Executive Summary & Ingestion Overview

This official report consolidates the automated post-earthquake Building Vulnerability Assessment for structures impacted by the 2001 Bhuj Earthquake. Utilizing retrieval-augmented generation (RAG) grounded in field investigation team reports (EEFIT, EERI, IITK-NICEE), physical buildings were ingested, assigned deterministic cryptographic identifiers, and screened against the National Disaster Management Authority (NDMA) Rapid Visual Screening (RVS) Primer.

Total Unique Physical Buildings Validated: 81 (post-deduplication).

Duplicate Records Identified & Removed: 9 duplicate records removed via deterministic SHA-256 hashing.

Vector Corpus Filtering: 1,732 report text chunks isolated specifically for the Bhuj earthquake event.

Primary Structural Failure Modes: Extreme prevalence of soft/open ground storeys, floating columns, torsional irregularity from rooftop swimming pools, and out-of-plane masonry wall separations.

# 2. Building Breakdown by RVS Typology

All screened buildings were categorized under standard NDMA RVS typologies using rule-based classification and contextual keyword fallback search for buildings with incomplete initial metadata:

| RVS Typology | Description / Structural System | Building Count | Corpus Share (%) |
| --- | --- | --- | --- |
| RC | Reinforced Concrete Frame (Open Storey / Infill / Shear Wall) | 53 | 65.4% |
| RRSM | Random Rubble Stone Masonry (Mud / Cement Mortar) | 25 | 30.9% |
| BCBM | Burnt Clay Brick Masonry (Load-bearing walls & industrial chimneys) | 3 | 3.7% |

Figure 1: Building distribution across screened RVS typologies.

# 3. Typology-Wise Detailed Vulnerability & Frequency Analysis

## Typology RC: Reinforced Concrete Frame (Open Storey / Infill / Shear Wall) (53 Buildings)

Detailed breakdown of observed NDMA RVS parameters and additional post-earthquake failure mechanisms identified across the 53 buildings of typology RC:

| Source | Parameter ID | Parameter Description / Vulnerability | Observed | Typology % |
| --- | --- | --- | --- | --- |
| RVS Primer | RC_RVS_3.4 | Building has an Open Storey (at ground /other level) with slender columns. | 24 | 45.3% |
| RVS Primer | RC_POST_3.3 | Building has an Open storey (at ground /other level) with columns having shear cracks. | 24 | 45.3% |
| Additional | ADD_RC_03 | Rooftop swimming pool causing extreme mass irregularity and asymmetric torsional collapse | 9 | 17.0% |
| Additional | ADD_GEN_01 | Common shared wall between adjacent buildings with differential response | 9 | 17.0% |
| Additional | ADD_RC_05 | Collapse of intermediate storey in commercial RC frame | 8 | 15.1% |
| Additional | ADD_RC_02 | Lack of core confinement / 90-degree ties with excessive longitudinal spacing | 5 | 9.4% |
| RVS Primer | RC_POST_3.9 | Building has extensive X cracking or out-of-plane collapse of infills. | 5 | 9.4% |
| RVS Primer | RC_RVS_8.2 | Building has RC walls unsymmetrical in both directions. | 5 | 9.4% |
| RVS Primer | RC_RVS_3.6 | Building has irregular plan geometry and or plan aspect ratio > 4.0. | 4 | 7.5% |
| Additional | ADD_RC_04 | Pancake collapse of multi-storey educational frame structure | 4 | 7.5% |
| RVS Primer | RC_RVS_8.4 | Building has unanchored water tanks on roof. | 4 | 7.5% |
| RVS Primer | RC_POST_5.2 | Building has water tanks on roof displaced from their supports. | 4 | 7.5% |
| RVS Primer | RC_RVS_9.1 | Building has longitudinal cracks and/or spalling in Beams and Columns. | 4 | 7.5% |
| Additional | ADD_RC_08 | Improper post-earthquake column jacketing not extended to foundations or beam joints | 4 | 7.5% |
| Additional | ADD_RC_06 | Dislodging of weakly connected overhead water tank appendages | 3 | 5.7% |
| RVS Primer | RC_RVS_3.5 | Building has RC frame with Floating Columns. | 2 | 3.8% |
| RVS Primer | RC_RVS_7.3 | Building has large or heavy cantilever balconies or sunshades. | 2 | 3.8% |
| RVS Primer | RC_POST_3.4 | Building has Floating Columns with cracked supporting beams. | 2 | 3.8% |
| RVS Primer | RC_POST_3.5 | Building has main load resisting columns with shear cracks. | 2 | 3.8% |
| RVS Primer | RC_POST_5.1 | Building has sustained collapse of cantilevers, balconies, chimneys and parapets. | 2 | 3.8% |
| Additional | ADD_RC_10 | Insufficient connection between RC elevator core wall and adjacent framing | 2 | 3.8% |
| RVS Primer | RC_POST_2.3 | Building is resting on liquefied soil. | 2 | 3.8% |
| Additional | ADD_RC_01 | Inadequate lap length in longitudinal column reinforcement | 1 | 1.9% |
| Additional | ADD_RC_09 | Shear cracks in cantilever stub beam supporting floating columns | 1 | 1.9% |
| Additional | ADD_RC_11 | Dislodgement of precast roof panels atop precast frame due to inadequate connections | 1 | 1.9% |
| Additional | ADD_GEN_02 | Failure of welded tie connections between prestressed concrete trusses | 1 | 1.9% |
| Additional | ADD_RC_07 | Temporary steel section or wooden post propping of damaged open ground storeys | 1 | 1.9% |

Figure: Parameter frequency distribution for typology RC.

## Typology RRSM: Random Rubble Stone Masonry (Mud / Cement Mortar) (25 Buildings)

Detailed breakdown of observed NDMA RVS parameters and additional post-earthquake failure mechanisms identified across the 25 buildings of typology RRSM:

| Source | Parameter ID | Parameter Description / Vulnerability | Observed | Typology % |
| --- | --- | --- | --- | --- |
| RVS Primer | RRSM_POST_5.8 | Building has floor-wall junction separated with walls out-of-plumb. | 5 | 20.0% |
| Additional | ADD_RRSM_01 | Out-of-plane collapse of stone masonry wall | 3 | 12.0% |
| RVS Primer | RRSM_RVS_4.3 | Building has no continuous lintel band, roof band and gable band. | 1 | 4.0% |
| RVS Primer | RRSM_RVS_4.5 | Building has masonry walls not integrated into each other at corners. | 1 | 4.0% |
| RVS Primer | RRSM_POST_5.4 | Building has collapsed/damaged staircase/mumty or blockade of staircase. | 1 | 4.0% |
| RVS Primer | RRSM_POST_5.7 | Building has suffered separation of wall junctions at corners. | 1 | 4.0% |
| Additional | ADD_RRSM_02 | Collapse of staircase and building corner in multi-storey stone masonry | 1 | 4.0% |
| RVS Primer | RRSM_RVS_4.6 | Building has walls made with no or mud mortar exposed to vagaries. | 1 | 4.0% |
| RVS Primer | RRSM_RVS_5.1 | Building has walls made with mud mortar and deteriorated and cracked significantly owing to vagaries of outside weather (especially rainwater beating). | 1 | 4.0% |

Figure: Parameter frequency distribution for typology RRSM.

## Typology BCBM: Burnt Clay Brick Masonry (Load-bearing walls & industrial chimneys) (3 Buildings)

Detailed breakdown of observed NDMA RVS parameters and additional post-earthquake failure mechanisms identified across the 3 buildings of typology BCBM:

| Source | Parameter ID | Parameter Description / Vulnerability | Observed | Typology % |
| --- | --- | --- | --- | --- |
| RVS Primer | BCBM_POST_3.3 | Building has plinth masonry severely damaged. | 1 | 33.3% |
| RVS Primer | BCBM_RVS_4.9 | Building has cracks in walls. | 1 | 33.3% |
| RVS Primer | BCBM_POST_3.9 | Building has diagonal shear cracking in wall piers and/or spandrels. | 1 | 33.3% |
| RVS Primer | BCBM_POST_5.1 | Building has sustained collapse of cantilevers, balconies, chimneys and parapets. | 1 | 33.3% |
| Additional | ADD_BCBM_01 | Cyclic shear cracks in industrial brick masonry chimney | 1 | 33.3% |

Figure: Parameter frequency distribution for typology BCBM.

# 4. Additional Earthquake-Observed Parameters

A critical strength of this pipeline is its ability to extract structurally meaningful phenomena documented during field investigations that are not explicitly cataloged in baseline pre-earthquake RVS scoring manuals. These parameters represent critical vulnerabilities observed in the Bhuj event:

[BCBM] ADD_BCBM_01: Cyclic shear cracks in industrial brick masonry chimney — Observed in 1 building(s) (33.3%)

[RC] ADD_RC_03: Rooftop swimming pool causing extreme mass irregularity and asymmetric torsional collapse — Observed in 9 building(s) (17.0%)

[RC] ADD_GEN_01: Common shared wall between adjacent buildings with differential response — Observed in 9 building(s) (17.0%)

[RC] ADD_RC_05: Collapse of intermediate storey in commercial RC frame — Observed in 8 building(s) (15.1%)

[RC] ADD_RC_02: Lack of core confinement / 90-degree ties with excessive longitudinal spacing — Observed in 5 building(s) (9.4%)

[RC] ADD_RC_04: Pancake collapse of multi-storey educational frame structure — Observed in 4 building(s) (7.5%)

[RC] ADD_RC_08: Improper post-earthquake column jacketing not extended to foundations or beam joints — Observed in 4 building(s) (7.5%)

[RC] ADD_RC_06: Dislodging of weakly connected overhead water tank appendages — Observed in 3 building(s) (5.7%)

[RC] ADD_RC_10: Insufficient connection between RC elevator core wall and adjacent framing — Observed in 2 building(s) (3.8%)

[RC] ADD_RC_01: Inadequate lap length in longitudinal column reinforcement — Observed in 1 building(s) (1.9%)

[RC] ADD_RC_09: Shear cracks in cantilever stub beam supporting floating columns — Observed in 1 building(s) (1.9%)

[RC] ADD_RC_11: Dislodgement of precast roof panels atop precast frame due to inadequate connections — Observed in 1 building(s) (1.9%)

[RC] ADD_GEN_02: Failure of welded tie connections between prestressed concrete trusses — Observed in 1 building(s) (1.9%)

[RC] ADD_RC_07: Temporary steel section or wooden post propping of damaged open ground storeys — Observed in 1 building(s) (1.9%)

[RRSM] ADD_RRSM_01: Out-of-plane collapse of stone masonry wall — Observed in 3 building(s) (12.0%)

[RRSM] ADD_RRSM_02: Collapse of staircase and building corner in multi-storey stone masonry — Observed in 1 building(s) (4.0%)